MSDS Number: F1368 * * * * * Effective Date: 01/19/06 * * * * * Supercedes: 08/10/04

# FERRIC SULFATE

## 1. Product Identification

Synonyms: Iron (III) sulfate; iron persulfate; sulfuric acid, iron (3+) salt (3:2) hydrate 
CAS No.: 10028-22-5 (Anhydrous); 15244-10-7 (Hydrated) 
Molecular Weight: 399.87 
Chemical Formula: Fe2(SO4)3. xH2O 
Product Codes: 
J.T. Baker: 2046 
Mallinckrodt: 5036

## 2. Composition/Information on Ingredients

Ingredient                                CAS No         Percent        Hazardous

---------------------------------------   ------------   ------------   ---------

Ferric Sulfate                            10028-22-5       70 - 80%        Yes

Water                                     7732-18-5        20 - 30%        No

## 3. Hazards Identification

Emergency Overview 
-------------------------- 
WARNING! HARMFUL IF SWALLOWED OR INHALED. CAUSES IRRITATION TO SKIN, EYES AND RESPIRATORY TRACT. AFFECTS THE LIVER. 

SAF-T-DATA(tm) Ratings (Provided here for your convenience) 
----------------------------------------------------------------------------------------------------------- 
Health Rating: 3 - Severe (Life) 
Flammability Rating: 0 - None 
Reactivity Rating: 0 - None 
Contact Rating: 2 - Moderate 
Lab Protective Equip: GOGGLES; LAB COAT; VENT HOOD; PROPER GLOVES 
Storage Color Code: Green (General Storage) 
----------------------------------------------------------------------------------------------------------- 

Potential Health Effects 
---------------------------------- 

Inhalation: 
Causes irritation to the respiratory tract. Symptoms may include coughing, shortness of breath. 
Ingestion: 
Low toxicity in small quantities but larger dosages may cause nausea, vomiting, diarrhea, and black stool. Pink urine discoloration is a strong indicator of iron poisoning. Liver damage, coma, and death from iron poisoning has been recorded. 
Skin Contact: 
Causes irritation to skin. Symptoms include redness, itching, and pain. May cause skin discoloration with irritation. 
Eye Contact: 
Causes irritation, redness, and pain. 
Chronic Exposure: 
Prolonged exposure of the eyes may cause discoloration. Repeated high exposure could cause too much iron to build up in the body. Symptoms of upset stomach, nausea, constipation and black bowel movements may occur. Chronic exposure may cause liver effects. 
Aggravation of Pre-existing Conditions: 
Persons with pre-existing skin disorders or eye problems, or impaired liver, kidney or respiratory function may be more susceptible to the effects of the substance.

## 4. First Aid Measures

Inhalation: 
Remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Get medical attention. 
Ingestion: 
Induce vomiting immediately as directed by medical personnel. Never give anything by mouth to an unconscious person. Get medical attention. 
Skin Contact: 
Immediately flush skin with plenty of soap and water for at least 15 minutes. Remove contaminated clothing and shoes. Get medical attention. Wash clothing before reuse. Thoroughly clean shoes before reuse. 
Eye Contact: 
Immediately flush eyes with plenty of water for at least 15 minutes, lifting lower and upper eyelids occasionally. Get medical attention immediately.

## 5. Fire Fighting Measures

Fire: 
Not considered to be a fire hazard. 
Explosion: 
Not considered to be an explosion hazard. 
Fire Extinguishing Media: 
Dry chemical, foam, carbon dioxide, or water spray. 
Special Information: 
In the event of a fire, wear full protective clothing and NIOSH-approved self-contained breathing apparatus with full facepiece operated in the pressure demand or other positive pressure mode.

## 6. Accidental Release Measures

Ventilate area of leak or spill. Wear appropriate personal protective equipment as specified in Section 8. Spills: Sweep up and containerize for reclamation or disposal. Vacuuming or wet sweeping may be used to avoid dust dispersal. For ferric chloride anhydrous: US Regulations (CERCLA) require reporting spills and releases to soil, water and air in excess of reportable quantities. The toll free number for the US Coast Guard National Response Center is (800) 424-8802.

## 7. Handling and Storage

Keep in a tightly closed light-resistant container, stored in a cool, dry, ventilated area. Protect against physical damage. Containers of this material may be hazardous when empty since they retain product residues (dust, solids); observe all warnings and precautions listed for the product.

## 8. Exposure Controls/Personal Protection

Airborne Exposure Limits: 
-ACGIH Threshold Limit Value (TLV):
1 mg/m3 (TWA) soluble iron salt as Fe 
Ventilation System: 
A system of local and/or general exhaust is recommended to keep employee exposures below the Airborne Exposure Limits. Local exhaust ventilation is generally preferred because it can control the emissions of the contaminant at its source, preventing dispersion of it into the general work area. Please refer to the ACGIH document, Industrial Ventilation, A Manual of Recommended Practices, most recent edition, for details. 
Personal Respirators (NIOSH Approved): 
If the exposure limit is exceeded and engineering controls are not feasible, a half facepiece particulate respirator (NIOSH type N95 or better filters) may be worn for up to ten times the exposure limit or the maximum use concentration specified by the appropriate regulatory agency or respirator supplier, whichever is lowest.. A full-face piece particulate respirator (NIOSH type N100 filters) may be worn up to 50 times the exposure limit, or the maximum use concentration specified by the appropriate regulatory agency, or respirator supplier, whichever is lowest. If oil particles (e.g. lubricants, cutting fluids, glycerine, etc.) are present, use a NIOSH type R or P filter. For emergencies or instances where the exposure levels are not known, use a full-facepiece positive-pressure, air-supplied respirator. WARNING: Air-purifying respirators do not protect workers in oxygen-deficient atmospheres. 
Skin Protection: 
Wear impervious protective clothing, including boots, gloves, lab coat, apron or coveralls, as appropriate, to prevent skin contact. 
Eye Protection: 
Maintain eye wash fountain and quick-drench facilities in work area. Use chemical safety goggles and/or full face shield where dusting or splashing of solutions is possible.

## 9. Physical and Chemical Properties

Appearance: 
Grayish-white powder or rhombic crystals. 
Odor: 
Odorless. 
Solubility: 
Soluble in water. 
Density: 
3.097 (Anhydrous) 
pH: 
No information found. 
% Volatiles by volume @ 21C (70F): 
0 
Boiling Point: 
Not applicable. 
Melting Point: 
480C (896F) 
Vapor Density (Air=1): 
No information found. 
Vapor Pressure (mm Hg): 
No information found. 
Evaporation Rate (BuAc=1): 
No information found.

## 10. Stability and Reactivity

Stability: 
Stable under ordinary conditions of use and storage. Very hygroscopic. 
Hazardous Decomposition Products: 
Oxides of sulfur and the contained metal. 
Hazardous Polymerization: 
This substance does not polymerize. 
Incompatibilities: 
No incompatibility data found. 
Conditions to Avoid: 
Heat, light, moisture.

## 11. Toxicological Information

Oral rat LD50: 500 mg/kg. Investigated as a mutagen.

--------\Cancer Lists\------------------------------------------------------

---NTP Carcinogen---

Ingredient                             Known    Anticipated    IARC Category

------------------------------------   -----    -----------    -------------

Ferric Sulfate (10028-22-5)             No          No            None

Water (7732-18-5)                       No          No            None

## 12. Ecological Information

Environmental Fate: 
No information found. 
Environmental Toxicity: 
No information found.

## 13. Disposal Considerations

Whatever cannot be saved for recovery or recycling should be managed in an appropriate and approved waste disposal facility. Processing, use or contamination of this product may change the waste management options. State and local disposal regulations may differ from federal disposal regulations. Dispose of container and unused contents in accordance with federal, state and local requirements.

## 14. Transport Information

Not regulated.

## 15. Regulatory Information

--------\Chemical Inventory Status - Part 1\---------------------------------

Ingredient                                       TSCA  EC   Japan  Australia

-----------------------------------------------  ----  ---  -----  ---------

Ferric Sulfate (10028-22-5)                       Yes  Yes   Yes      Yes

Water (7732-18-5)                                 Yes  Yes   Yes      Yes

--------\Chemical Inventory Status - Part 2\---------------------------------

--Canada--

Ingredient                                       Korea  DSL   NDSL  Phil.

-----------------------------------------------  -----  ---   ----  -----

Ferric Sulfate (10028-22-5)                       Yes   Yes   No     Yes

Water (7732-18-5)                                 Yes   Yes   No     Yes

--------\Federal, State & International Regulations - Part 1\----------------

-SARA 302-    ------SARA 313------

Ingredient                                 RQ    TPQ     List  Chemical Catg.

-----------------------------------------  ---   -----   ----  --------------

Ferric Sulfate (10028-22-5)                No    No      No         No

Water (7732-18-5)                          No    No      No         No

--------\Federal, State & International Regulations - Part 2\----------------

-RCRA-    -TSCA-

Ingredient                                 CERCLA     261.33     8(d)

-----------------------------------------  ------     ------    ------

Ferric Sulfate (10028-22-5)                1000       No         No

Water (7732-18-5)                          No         No         No

Chemical Weapons Convention:  No     TSCA 12(b):  No     CDTA:  No

SARA 311/312:  Acute: Yes      Chronic: Yes  Fire: No  Pressure: No

Reactivity: No          (Mixture / Solid)

Australian Hazchem Code: None allocated. 
Poison Schedule: None allocated. 
WHMIS: 
This MSDS has been prepared according to the hazard criteria of the Controlled Products Regulations (CPR) and the MSDS contains all of the information required by the CPR.

## 16. Other Information

NFPA Ratings: Health: 1 Flammability: 0 Reactivity: 0 
Label Hazard Warning: 
WARNING! HARMFUL IF SWALLOWED OR INHALED. CAUSES IRRITATION TO SKIN, EYES AND RESPIRATORY TRACT. AFFECTS THE LIVER. 
Label Precautions: 
Avoid contact with eyes, skin and clothing.
Wash thoroughly after handling.
Avoid breathing dust.
Keep container closed.
Use only with adequate ventilation. 
Label First Aid: 
In case of contact, immediately flush eyes or skin with plenty of water for at least 15 minutes. Remove contaminated clothing and shoes. Wash clothing before reuse. If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. If swallowed, induce vomiting immediately as directed by medical personnel. Never give anything by mouth to an unconscious person. In all cases, get medical attention. 
Product Use: 
Laboratory Reagent. 
Revision Information: 
MSDS Section(s) changed since last revision of document include: 3, 11. 
Disclaimer: 
************************************************************************************************ 
Mallinckrodt Baker, Inc. provides the information contained herein in good faith but makes no representation as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose. MALLINCKRODT BAKER, INC. MAKES NO REPRESENTATIONS OR WARRANTIES, EITHER EXPRESS OR IMPLIED, INCLUDING WITHOUT LIMITATION ANY WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE WITH RESPECT TO THE INFORMATION SET FORTH HEREIN OR THE PRODUCT TO WHICH THE INFORMATION REFERS. ACCORDINGLY, MALLINCKRODT BAKER, INC. WILL NOT BE RESPONSIBLE FOR DAMAGES RESULTING FROM USE OF OR RELIANCE UPON THIS INFORMATION. 
************************************************************************************************ 
Prepared by: Environmental Health & Safety
Phone Number: (314) 654-1600 (U.S.A.)