VALVOLINE OIL -- NAPA HFC 134-A REFRIGERANT -- -

=====================  Product Identification  =====================

Product ID:NAPA HFC 134-A REFRIGERANT

MSDS Date:02/10/1995

FSC:NIIN:Submitter:F BT

Status Code:A

MSDS Number: CJSBS

=== Responsible Party ===

Company Name:VALVOLINE OIL

Box:14000

City:LEXINGTON

State:KY

ZIP:40512

Country:US

Info Phone Num:606-357-7847

Emergency Phone Num:800-274-5263

CAGE:72527

=== Contractor Identification ===

Company Name:VALVOLINE OIL CO

Address:3499 BLAZER PKY

Box:14000

City:LEXINGTON

State:KY

ZIP:40512

Country:US

Phone:606-357-7206 / 800-274-5263

CAGE:72527

=============  Composition/Information on Ingredients  =============

Ingred Name:ETHANE, 1,1,1,2-TETRAFLUORO-

CAS:811-97-2

= Wt:100.

=====================  Hazards Identification  =====================

Routes of Entry: Inhalation:YES  Skin:YES  Ingestion:YES

Health Hazards Acute and Chronic:EYES: CAUSES IRRITATION. SKIN:

BURNS/FROSTBITE. INGESTION: SINGLE DOSE ORAL TOXICITY IS LOW.

SWALLOWING LARGE AMOUNTS MAY BE HARMFUL. GI IRRITANT.  INHALATION:

SHORT TERM TOXICITY IS LOW. BREATHING LA RGE AMOUNTS MAY BE

HARMFUL. IRRITATION TO NOSE, THROAT & RESPIRATORY TRACT. MAY CAUSE

CNS DEPRESSION & DEATH.

Effects of Overexposure:IRRITATION, STINGING, TEARING, REDNESS,

SWELLING, BURNS, FROSTBITE, NAUSEA, VOMITING, DIARRHEA, DIZZINESS,

DROWSINESS, WEAKNESS, FATIGUE, HEADACHE, UNCONSCIOUSNESS

Medical Cond Aggravated by Exposure:PRE-EXISTING LUNG DISORDERS,

ASTHMA-LIKE CONDITIONS.

=======================  First Aid Measures  =======================

First Aid:EYES: IMMEDIATELY FLUSH GENTLY W/WATER FOR 15 MINS.

IMMEDIATELY MOVE TO FRESH AIR BEFORE FLUSHING. SKIN: TREAT

BURNED/FROSTBITE BY FLUSHING W/LUKEWARM WATER. INGESTION: DON'T

INDUCE VOMITING. ASPIRATI ON HAZARD. IF DROWSY/UNCONSCIOUS, PLACE

ON LEFT SIDE W/HEAD DOWN.  DON'T LEAVE INDIVIDUAL UNATTENDED.

INHALATION: MOVE AWAY FROM EXPOSURE INTO FRESH AIR. KEEP

WARM/QUIET. GIVE CPR/OXYGEN IF NECESSARY.  OBTAIN MEDICAL ATTENTION

IN ALL CASES. NOTE TO PHYSICIAN: INHALATION OF HIGH CONCENTRATIONS

COULD OCCUR IN ENCLOSED SPACES & MAY BE ASSOCIATED W/CARDIAC

ARRHYTHMIAS.

=====================  Fire Fighting Measures  =====================

Extinguishing Media:WATER FOG

Fire Fighting Procedures:WATER MAY BE USED TO KEEP FIRE-EXPOSED

CONTAINERS COOL UNTIL FIRE IS OUT. WEAR A SELF CONTAINED BREATHING

APPARATUS W/A FULL FACEPIECE OPERATED IN THE POSITIVE PRESSURE

DEMAND MODE W/APPROPRIATE TURN- OUT GEAR & CHEMICAL RESISTANT

PERSONAL PROTECTIVE EQUIPMENT.

==================  Accidental Release Measures  ==================

Spill Release Procedures:SMALL: ALLOW TO EVAPORATE. VENTILATE AREA.

LARGE : ALLOW TO EVAPORATE. PERSONS NOT WEARING PROTECTIVE

EQUIPMENT SHOULD BE EXCLUDED FROM AREA UNTIL LEAK HAS BEEN

REPAIRED.

======================	Handling and Storage  ======================

Handling and Storage Precautions:CONTAINERS OF THIS MATERIAL MAY BE

HAZARDOUS WHEN EMPTIED. SINCE EMPTIED CONTAINERS RETAIN PRODUCT

RESIDUES; VAPOR/LIQUID/SOLID, ALL HAZARD PRECAUTIONS GIVEN IN THE

DATA SHEET MUST BE OBSERVED.

=============  Exposure Controls/Personal Protection  =============

Respiratory Protection:IF WORKPLACE EXPOSURE LIMITS OF PRODUCT IS

EXCEEDED, A NIOSH/MSHA APPROVED AIR SUPPLIED RESPIRATOR IS ADVISED

IN ABSENCE OF PROPER ENVIRONMENTAL CONTROL.

Ventilation:SUFFICIENT MECHANICAL GENERAL/LOCAL EXHAUST VENTILATION TO

MAINTAIN EXPOSURE BELOW TLVS.

Protective Gloves:POLYVINYL ALCOHOL

Eye Protection:CHEMICAL SPLASH GOGGLES IN COMPLIANCE W/OSHA

REGULATIONS.

Other Protective Equipment:WORK CLOTHING COVERING ARMS & LEGS

Supplemental Safety and Health

==================  Physical/Chemical Properties  ==================

Boiling Pt:=-8.9C, 15.7F

Vapor Pres:4964

Spec Gravity:1.21

Appearance and Odor:PRODUCT IS A REFRIGERANT

=================  Stability and Reactivity Data  =================

Stability Indicator/Materials to Avoid:YES

ALKALI METALS, POWDERED METALS

===================  Toxicological Information	===================

Toxicological Information:NO DATA

=====================  Ecological Information  =====================

Ecological:NO DATA

====================  Disposal Considerations  ====================

Waste Disposal Methods:RETURN TO SUPPLIER FOR RECLAMATION. DISPOSE OF

IN ACCORDANCE W/LOCAL, STATE & FEDERAL REGULATIONS.

===================  MSDS Transport Information  ===================

Transport Information:DOT INFORMATION- 49 CFR 172.101. DOT DESCRIPTION:

1,1,1,2-TETRAFLUOROETHANE 2,2 UN3159.

=====================  Regulatory Information  =====================

SARA Title III Information:SECTION 311/312 HAZARD CLASS: IMMEDIATE,

DELAYED.

Federal Regulatory Information:NONE

State Regulatory Information:NONE

=======================  Other Information  =======================

Disclaimer (provided with this information by the compiling agencies):

This information is formulated for use by elements of the Department

of Defense.  The United States of America in no manner whatsoever,

expressly or implied, warrants this information to be accurate and

disclaims all liability for its use.  Any person utilizing this

document should seek competent professional advice to verify and

assume responsibility for the suitability of this information to their

particular situation.